# ASSESSMENT TOOL 2

Case Study

## Code of Conduct

I  J.Vishnu Vardhan-(192324297) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action. Signature of the student: J.Vishnu Vardhan

## Rubric for Calculation

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Case | 25 | Thorough understanding; clearly identifies all key issues | Good understanding with most issues identified | Basic understanding; some issues missed | Poor understanding; problem not identified correctly |
| Application of Concepts | 25 | Effectively applies relevant concepts to analyze the case deeply | Applies appropriate concepts with minor gaps | Limited application of concepts | Fails to apply relevant concepts |
| Analysis | 20 | In-depth analysis with strong reasoning and insights | Good analysis with logical reasoning | Basic analysis with limited depth | Weak or no analysis; lacks reasoning |
| Solution Design | 20 | Innovative, feasible solutions with strong justification | Logical solutions with adequate justification | Basic solutions with weak justification | Incorrect or no solution provided |
| Clarity | 10 | Clear, well-structured, and easy to understand | Organized and understandable presentation | Limited clarity and structure | Poor clarity; difficult to understand |

### Case Study 1: Defect Detection in Industrial Parts

A manufacturing company wants to automatically detect defects in gears and machine components using computer vision. The images of the components are binary and may contain small protrusions or missing parts.

### Q1(A). How can region-based representation and medial axis computation help in identifying defective parts?

Answer:

- Region-based representation: A gear or component is described using every pixel enclosed by its boundary rather than only the outline. Descriptors such as area, perimeter-to-area ratio, Euler number (number of holes), centroid, and moments (including Hu moments) are computed for the binary blob. A defective part — one with a broken tooth, a missing chunk, or an extra protrusion (flash/burr) — shows a measurable deviation in these region descriptors when compared against the descriptors of a known-good "golden" reference part of the same type.

- Medial Axis Transform (MAT): The medial axis (skeleton) is the set of points inside the shape that have more than one closest point on the boundary, typically computed from the distance transform followed by thinning. For a gear, the skeleton captures the underlying topology — the hub, the spokes/branches corresponding to each tooth, their length, angle, and symmetry. Because the MAT (skeleton plus the distance-to-boundary value at each skeleton point) is invertible, it is a compact but lossless description of the region's shape.

- How they help detect defects: A missing tooth removes a branch from the skeleton graph, while a protrusion or burr introduces a spurious short branch that is not present in the reference skeleton. Comparing the number, length, and angular spacing of skeleton branches against the golden template pinpoints exactly which region of the part is defective. Combining coarse region descriptors (fast rejection of grossly defective parts) with skeleton/topology analysis (fine localization of the defect) gives both a quick pass/fail decision and a precise defect location, which is valuable for automated visual inspection on a production line.

### Q1(B). Discuss how noise in image acquisition could affect shape analysis and propose preprocessing steps to improve accuracy.

Answer:

Effect of noise on shape analysis:

- Salt-and-pepper / sensor noise can flip isolated pixels in the binary image, creating tiny spurious connected components or small holes, which are then wrongly interpreted as missing material or extra protrusions.

- Noise on the object boundary distorts contour-based descriptors — chain codes, boundary curvature, and Fourier descriptors all become noisy, since these methods are sensitive to small perturbations along the outline.

- Noise adds spurious short branches to the computed skeleton (medial axis), which can be misread as real structural defects, increasing false-positive defect detections.

- Uneven or inconsistent illumination during acquisition causes an unreliable binarization threshold, so regions may incorrectly merge or split, corrupting area, moment, and Euler-number calculations.

Proposed preprocessing steps:

- Denoising: apply a median filter (effective against salt-and-pepper noise) or a Gaussian filter before thresholding, to remove isolated noisy pixels without excessively blurring true edges.

- Illumination correction: use background subtraction, homomorphic filtering, or adaptive/local (rather than global) thresholding such as adaptive Otsu, so lighting variation across the field of view does not shift the binarization boundary.

- Morphological cleanup: apply opening (erosion followed by dilation) to remove small spurious protrusions and closing to fill small unwanted holes, using a small structuring element so genuine small defects are not erased along with the noise.

- Connected-component filtering: run connected-component analysis and discard blobs below a minimum area threshold, since these are almost always noise rather than real defects.

- Contour smoothing: smooth the extracted boundary (e.g. Gaussian smoothing or polygonal approximation such as Douglas–Peucker) before computing curvature or Fourier descriptors, to suppress high-frequency noise components while preserving the genuine shape.

- Skeleton pruning: after computing the medial axis, prune branches shorter than a minimum-length threshold, since these are typically artifacts of boundary noise rather than true structural features.

### Q1(C). Compare the effectiveness of Fourier descriptors versus wavelet descriptors for classifying normal vs. defective components.

Answer:

- Fourier Descriptors (FD): obtained by taking the Discrete Fourier Transform of the boundary parametrized as a sequence of complex coordinates (x + iy). Low-order coefficients capture the coarse, global shape (e.g. the overall circular envelope of a gear) while higher-order coefficients capture finer detail. FD can be normalized to be invariant to translation, rotation, scale, and starting point, and they give a compact global descriptor that is well suited to classifying overall part type or gross shape category.

- Wavelet Descriptors (WD): obtained by decomposing the boundary signal using a wavelet basis, which is localized in both position and frequency (multi-resolution analysis). This means a defect at one specific location on the boundary (a chipped tooth, a small nick) affects only the wavelet coefficients local to that region of the boundary, rather than being spread across the whole coefficient set as with a global Fourier basis.

- Comparison for this case: since the defects described are small, localized protrusions or missing sections, wavelet descriptors are generally more effective for classifying normal vs. defective components because their locality lets them both detect and localize a small anomaly using only a few coefficients, and their multi-resolution nature lets coarse shape and fine defect detail be analyzed together. Fourier descriptors, by contrast, need a large number of high-order coefficients to represent a small localized irregularity, are more sensitive to noise being spread across all coefficients, and are comparatively better suited to distinguishing broad shape categories than to catching a single small defect.

- Practical recommendation: a hybrid pipeline — Fourier descriptors for fast, coarse classification of part type/gross shape, followed by wavelet descriptors for fine-grained normal-vs-defective classification — combines the strengths of both.

### Case Study 2: Medical Image Segmentation

A hospital is developing a system to segment organs from CT scans. The organ boundaries are irregular and may vary slightly between patients.

### Q2(A). Explain how active contours (snakes) can be applied to segment the organ boundaries.

Answer:

An active contour, or "snake", is a deformable curve v(s) = (x(s), y(s)) that evolves within the image to minimize an energy functional:

E(v) = ∫ [ Internal energy(v) + External/Image energy(v) ] ds

- Internal energy controls the smoothness of the curve — an elasticity (first-order, continuity) term weighted by α and a rigidity/curvature (second-order, bending) term weighted by β — which keeps the contour continuous and prevents it from developing unrealistic sharp kinks.

- External (image) energy pulls the curve toward strong intensity gradients — typically defined as the negative of the squared gradient magnitude — so the curve is attracted to the actual organ boundary in the CT slice.

Applied to organ segmentation, the workflow is:

- Initialize a contour near the expected organ boundary, either manually by a clinician/technician or automatically from a coarse segmentation such as thresholding or region growing.

- Iteratively deform the contour (via gradient descent, the greedy algorithm, or dynamic programming) so that it moves toward nearby edges while the internal energy keeps it smooth.

- The snake converges when it reaches a local minimum of the energy functional, at which point it should coincide with the true, patient-specific organ boundary — even though that boundary is irregular and not a simple geometric shape.

Because the snake is a flexible, elastic curve rather than a fixed template, it naturally adapts to the irregular and patient-to-patient variable shape of organs such as the liver, kidney, or spleen in CT images.

### Q2(B). Discuss the challenges of initializing contours and tuning parameters for accurate segmentation.

Answer:

### Initialization challenges:

- A snake only has a limited "capture range" — the external force only pulls the curve toward edges that are reasonably close to it — so it must be initialized fairly near the true organ boundary, or it will converge to the wrong edge (e.g. a neighboring organ) or get stuck at a local minimum.

- Manual initialization on every CT slice of a 3D volume is time-consuming and impractical at scale; automatic initialization (thresholding, region growing, atlas-based priors) is faster but often imprecise, especially when organ intensity is similar to surrounding tissue.

- A common remedy is Gradient Vector Flow (GVF), which diffuses the edge-gradient information outward through the image so the snake can be attracted to boundaries from farther away, extending the effective capture range.

### Parameter tuning challenges:

- The elasticity weight α and rigidity weight β control the smoothness-versus-flexibility trade-off: too high, and the contour over-smooths, cutting across genuine concavities in an irregular organ boundary; too low, and the contour becomes noisy and can leak through weak or broken edges.

- The weight given to external (image) energy trades off against noise sensitivity: too strong, and the snake latches onto spurious edges caused by CT noise or an adjacent organ of similar density; too weak, and the snake fails to lock onto the true, sometimes low-contrast, organ boundary.

- Because CT contrast and organ shape vary between patients (and even between slices of the same patient), a parameter set tuned for one case often does not generalize well to another, requiring either per-case tuning or adaptive/learned parameter selection.

- Where the organ boundary touches another structure of similar intensity (a very common situation in abdominal CT), the external energy may be too weak or ambiguous at that boundary segment, causing the contour to leak across into the neighboring structure.

### Q2(C). How could level set methods improve segmentation for cases where organs split or merge in the images?

Answer:

- Limitation of the classical (parametric) snake: it is represented explicitly as an ordered list of boundary points with a fixed topology. It cannot naturally split into two separate contours (e.g. when an organ appears as two disconnected lobes in a slice) or merge two contours into one, without extra, error-prone reparametrization logic.

- Level set formulation: the contour is represented implicitly as the zero level set of a higher-dimensional function φ(x, y, t) — commonly a signed distance function — which evolves according to a partial differential equation such as φt + F|∇φ| = 0, where the speed term F is typically driven by boundary curvature and image-based edge/region information (as in geodesic active contours).

- Why this helps with splitting/merging: because the boundary is only the implicit zero-crossing of φ rather than an explicitly tracked curve, topological changes — the surface splitting into two separate zero-level curves, or two curves merging into one — happen automatically as φ evolves, with no special-case handling required. This makes level sets naturally suited to organs that branch into lobes, or CT slices where two structures appear to merge and then separate across the volume.

- Additional benefits: the method extends directly to full 3D volumetric segmentation (evolving a hypersurface rather than a 2D curve), handles sharp corners and cusps robustly, and geodesic active-contour formulations in the level set framework are more stable with respect to initialization than classical snakes.

- Trade-offs: level set evolution is computationally more expensive since the PDE is, in principle, solved over the whole image grid (mitigated in practice with narrow-band methods), φ typically needs periodic reinitialization to remain a valid distance function, and careful design of the speed function and curvature-versus-edge-term balance is still required for accurate convergence.